ANSWER 1

- Selected Sensor Architecture :-

Intel Realsense Depth Camera D435i and NVIDIA Jetson Orin Nano

- To determine the best autonomous architecture, the parameters are weighted based on their critical importance for real time robotics and navigation:-

- Compute (Weight: 35%): The NVIDIA Jetson Orin Nano provides up to 40 TOPS of AI performance, vastly outperforming standard Raspberry Pi CPU setups. This massive GPU accelerated compute is essential for running real time SLAM (Simultaneous Localization and Mapping), depth estimation, and neural network based object detection without latency.

- Use Case Compatibility (Weight: 25%): The Intel RealSense D435i combines an active infrared stereo depth sensor with an integrated Inertial Measurement Unit (IMU). Unlike passive stereo cameras, the D435i projects an invisible IR dot pattern to calculate precise depth data, making it ideal for robust indoor and outdoor navigation, obstacle avoidance, and handling texture less or low light environments.

- Power Consumption (Weight: 20%): The system operates at an estimated total power draw of approximately 15W, derived from the sum of the Jetson Orin Nano operating at roughly 10W under typical AI workloads and the RealSense D435i drawing about 1.5W to 5W depending on active IR usage. At a standard supply voltage of 5V, the total current draw is calculated as:

Total Current = Total Power/Voltage = 15W/5V = 3A

While this draws more power than a lightweight Raspberry Pi setup, the trade-off is heavily justified by the substantial leap in processing capability and sensor reliability required to keep an autonomous system stable.

- Weight (Weight: 10%): The hardware adds moderate weight, but it remains well within acceptable operational thresholds for standard mobile robotic platforms or larger drones, ensuring system agility is preserved.

- Cost (Weight: 10%): It sits at a higher price point than lower-end builds, but the investment is fully justified by eliminating the processing bottlenecks, latency issues, and failure rates associated with underpowered hardware.

ANSWER 2

- RC Transmitter and Receiver

- Uses: Enables manual remote piloting, flight mode switching, throttle/directional control, and manual failsafe execution.

- Model Selection: RadioMaster Boxer transmitter paired with an ExpressLRS (ELRS) receiver (such as the RadioMaster RP3). This digital setup entirely avoids legacy PWM/PPM signal degradation, channel limitations, and wiring clutter by using a high speed serial protocol.

- Connection: Connect the receiver's TX, RX, 5V, and GND pins to a designated UART (Serial) port on the Pixhawk 6C Mini for bidirectional CRSF communication.

- Power Source: Powered directly by the 5V output from the Pixhawk's serial/telemetry port or power rail.

- Protocol & Range: Communicates via the Crossfire (CRSF) protocol. Range > 10 km. The Range depends on RF output power (2.4 GHz or 915 MHz).

- Telemetry Module

- Uses: Streams real time flight data (GPS position, battery voltage, attitude, flight modes, and waypoint tracking) between the drone and a Ground Control Station (GCS) like Mission Planner or QGroundControl.

- Model Selection: Holybro SiK Telemetry Radio V3 (915MHz or 433MHz variant depending on regional regulations).

- Connection: Plugs directly into the TELEM1 or TELEM2 6-pin JST-GH port on the Pixhawk 6C Mini using the standard Pixhawk telemetry cable.

- Power Source: Powered directly by the 5V regulated output pin on the Pixhawk's TELEM port.

- Protocol & Range: MAVLink protocol packaged over radio frequency. Range varies from 1 km to 3 km with stock antennas (extendable with high gain antennas).

- VTX (Video Transmitter)

- Uses: Transmits real-time video feeds from the onboard camera to ground based FPV goggles or a monitor for visual navigation and inspection.

- Model Selection: DJI O3 Air Unit (Digital HD) or TBS Unify Pro32 HV (Analog). Selecting a modern digital system like the DJI O3 eliminates analog noise and low-resolution bottlenecks, providing clear HD monitoring.

- Connection: Connect power pads directly to the main power source, and wire the control/data lines to a spare UART on the flight computer or Pixhawk for OSD (On-Screen Display) telemetry overlay.

- Power Source: Powered directly from the main LiPo battery (via the power module's battery pads or a dedicated high-current 2S–6S input) because VTX units draw high current (up to several amps) that would overload the flight controller's internal regulator.

- Protocol & Range: Digital HD transmission or 5.8 GHz analog RF; Range: 1 km to 10 km+ depending on power output configuration (ranging from 25mW up to over 1000mW).

ANSWER 3

- Battery Selection:-

- Target Constraints:

- Maximum drone weight < 2.5kg

- Target endurance > 12 minutes

- Using a 4S LiPo configuration

- Hover Thrust & Power Estimation:

For a quadcopter with a total all up weight (AUW) of 2.2 kg (leaving 300g margin), each of the 4 motors must support a hover thrust of:

Thrust per Motor = 2200g/4 = 550g

Using a standard 1045 propeller on a 920KV motor at 4S voltage (approx. 14.8V), providing 550g of thrust typically consumes roughly 4.5A to 5.5A per motor. Taking a conservative hover current of 5A per motor:

Total Hover Current = 4 x 5A = 20A

Total Hover Power = Total Hover Current x Voltage = 20A x 14.8V = 296W

- Battery Capacity Calculation for 12+ Min Endurance:

To achieve a minimum of 12 minutes of flight time while factoring in a safe 80% Depth of Discharge (DoD), the required capacity (C) is calculated as:

C = (H x T)/SDF = (20 x 0.2)/0.8 = 5Ah = 5mAh

Where:-

C = Capacity

H = Hover Current in Ampere

T = Time in hours

Accounting for additional electronics load (flight computer, VTX, telemetry drawing approx. 1.5A total), the target capacity scales up to roughly 5200mAh to 6000mAh.

- Selected Battery Model: LiPo 4S 14.8V 5200mAh to 6000mAh (with a minimum 35C discharge rating).

Weight Check: A standard 4S 5200mAh LiPo weighs approximately 450g to 500g, keeping the total drone weight well around 2.15kg, safely under the 2.5kg maximum constraint.